This report presents the selection and justification for the mathematical modeling and simulation of a Lane Keeping Assist (LKA) system. The selected system has a monocular camera for line detection, a Linear Quadratic Regulator (LQR) control algorithm and a dynamic bicycle vehicle dynamics model. For the reference vehicle we have chosen the Toyota Corolla Altis X 1.6 Manual.

## Introduction

An LKA system is critical component of modern Driver Assisstance Systems (DAS). LKAs help to maintain the vehicles position within lane boundaries. The objective of this project is to develop a mathematical model and MATLAB simulation of an LKA system for autonomous vehicle steering control.

Module I will focus on selecting appropriate sensor technology, control algorithm, vehicle dynamics modeling and a reference vehicle, the technical specifications of which we can access.

## Sensor Technology Selection

For the sensor we had a couple of options, you will find a table below that provides a good overview of the options that were considered.

| Criteria | Monocular | Stereo | LiDAR | Radar |
| --- | --- | --- | --- | --- |
| Cost | Very Low | Low-Medium | Very High | Medium |
| Lane Detection | Excellent | Excellent | Indirect | Poor |
| Depth Perception | None | Good | Excellent | Good |
| Weather Robustness | Poor | Poor | Moderate | Excellent |
| MATLAB Modeling | Very Easy | Easy | Complex | Not Suitable |
| Academic Literature | Abundant | Good | Moderate | Limited |

Selected Sensor: The monocular camera has been selected.

#### Justification:

  - It outputs the lateral deviation and heading error directly, which are the inputs required for the control algorithm

  - It is used in MATLAB official LKA simulation examples which ensure compatibility and availability of reference material

  - There is an abundance of monocular lane detection algorithms with 95-97% accuracy.

Limitation: The sensor lacks depth perception and performs poorly in adverse weather conditions. Thus, this simulation assumes ideal daylight conditions on clear, well-marked roads, which is a standard academic assumption.

## Control Algorithm

You will find the comparative analysis of three different algorithms in the table below:

| Criteria | PID | LQR | MPC |
| --- | --- | --- | --- |
| Mathematical Complexity | Low | Medium | High |
| Tuning Parameters | 3 | Matrix Q, R | Model + Constraints |
| MATLAB Requirements | No toolbox | Control Toolbox | MPC Toolbox |
| LKA Performance | Good | Better | Best |
| Handles Constraints | No | No | Yes |
| Speed Robustness | Poor | Moderate | Excellent |
| Academic Literature | Abundant | Good | Abundant |

Selected Controller: LQR is the selected algorithm

#### Justification:

  - Provides mathematical optimal control by minimizing the quadratic cost function

∞

𝐽 = ∫

0

(𝑥𝑇𝑄𝑥 + 𝑢𝑇𝑅𝑢)𝑑𝑡

  - It is superior to PID for LKA as has been proven in literature and simpler to implement than MPC

  - Works seamlessly with dynamic bicycle model’s state variables

## Vehicle Dynamics Model

| Criteria | Kinematic Bicycle | Dynamic Bicycle |
| --- | --- | --- |
| Speed Range | >15 m/s | 5 – 25 m/s |
| Tire Slip Modeling | No | Yes |
| Inertial Effects | Negligible | Included |
| Parameters Required | 3 (L, 𝑙𝑓, 𝑙𝑟) | 7 (+ m, 𝐼𝑧, 𝐶𝑓, 𝐶𝑟) |
| MATLAB Support | Built-in block | Manual derivation |
| LKA Accuracy | Simplified | Realistic |
| Mathematical Form | Geometric | 2-DOF differential equations |

Selected: The dynamic bicycle model is selected.

#### Justification:

  - For highway speeds (15-25 m/s), this model is better suited.

  - Provides four natural state variables that integrate directly with LQR formulation

  - Explicitly recommended over kinematics models for lateral control at highway speeds in MATLAB documentation.

## Reference Vehicle Selection

Selected Vehicle: Toyota Corolla Altis X 1.6 manual

#### Justification:

  - Public availability of dimensions, mass, wheelbase and other characteristics required

  - Available locally, ensuring regional relevance

  - Fully capable of sustained highway speed operations

  - Based on well-documented vehicle dynamics in academic literature

#### Technical Specifications:

Below you find the technical specifications of the selected reference vehicle.

| Parameter | Value |
| --- | --- |
| Model Year | 2026 |
| Engine | 1.6L Dual VVT-i (1598 cc) |
| Power | 120 HP @ 6000 RPM |
| Torque | 154 Nm @ 5200 RPM |
| Kerb Weight (m) | 1275 kg |
| Wheelbase (L) | 2700 mm |
| Length × Width × Height | 4620 × 1775 × 1475 mm |
| Ground Clearance | 175 mm |
| Turning Radius | 5.4 m |
| Front Suspension | MacPherson Strut with Coil Springs |
| Rear Suspension | Torsion Beam with Coil Springs |
| Tire Size | 195/65 R15 |

## Conclusion

This report has established and justified the configuration for an LKA system’s mathematical modeling and MATLAB simulation. For a quick summary:

  - Sensor: Monocular Camera for direct line marking detection

  - Controller: LQR for optimal state feedback control

  - Dynamics Model: Dynamic bicycle model for highway speed accuracy

  - Reference Vehicle: 2026 Toyota Corolla Altis X 1.6 Manual

All assumptions have been mentioned in respective sections and justification provided. The configuration is grounded in academic literature and supported by MATLAB toolboxes. This completes our Module I report.